# Repeaters: Borrowing a Tall Tower

How one radio on a hill can stretch your range from 2 miles to 20 or more

## What a repeater does

- Your radio transmits: On a 467 MHz input channel (shown as 15R–22R)

- The tower hears you: Its antenna sits high, often 100 feet or more

- It retransmits instantly: On the matching 462 MHz channel, with more power

- Everyone in range hears it: Even family members miles beyond your handheld's reach

Why it works: Height is everything in radio. A repeater lends you its height.

## The numbers

- 5 MHz — gap between what you send and what you hear

- 8 — repeater channels (15R–22R)

- 20–50 — miles of typical coverage

- 50 W — maximum repeater power

Repeater inputs are for licensed GMRS users only; plain FRS walkie-talkies can't use them.

## Finding and using one

- Look it up: Search myGMRS.com or RepeaterBook for towers near you and the ranch

- Ask permission: Most repeaters are privately owned; the owner decides who may use them

- Set the tone: Repeaters need a specific access tone; the listing gives it

- Keep it short: Others share it; pause between messages so someone can break in

## The catch

### Great for everyday

- Family checks in across the county

- Works from inside a car or house

- No gear on your roof needed

### Don't count on it in a disaster

- Many towers lose power when the grid goes down

- Some are shut down or full of traffic

- Have a direct radio-to-radio plan too

## Remember

Use repeaters for range, but always have a radio-to-radio plan that works without one.