# How GPS Knows Where You Are

Satellites, atomic clocks and a little help from Einstein

## The system by the numbers

- 31 — working satellites, about

- 12,550 — miles up

- 2 — trips around Earth each day

- 4 — satellites needed for a fix

The system is run by the U.S. Space Force and is free for anyone in the world to use.

## How your phone finds itself

- Each satellite broadcasts the time: From an onboard atomic clock

- Your receiver notes when each signal arrives: The delay tells how far away the satellite is

- Three distances pin your position: Like circles that cross at one point

- A fourth fixes your own clock: Your phone's clock isn't accurate enough alone

## Einstein keeps GPS honest

### Two effects on the clocks

- Fast-moving clocks run slow (special relativity)

- Clocks in weaker gravity run fast (general relativity)

- Together, satellite clocks gain about 38 microseconds a day

### Why it matters

- Light travels about 7 miles in 38 microseconds

- Uncorrected, positions would drift about 6 miles a day

- Engineers set the satellite clocks slow before launch

## Remember

Every time you use a map app, you're using relativity.