# Why Height Beats Power

The earth curves, and GMRS signals travel in straight lines

## Radio waves travel in straight lines

### Line of sight

- GMRS works best when the antennas could almost see each other

- Hills, buildings and the earth's curve block the signal

- Trees and wet leaves soak it up

### What that means

- Doubling power adds only a little range

- Raising the antenna adds a lot

- A hilltop beats an extra 5 watts every time

## How far each antenna can 'see'

| Antenna height | Radio horizon | Example |
| --- | --- | --- |
| 6 ft | About 3.5 miles | Handheld at head height |
| 15 ft | About 5.5 miles | Second-story window |
| 30 ft | About 7.7 miles | Mast on the ranch house |
| 100 ft | About 14 miles | Tall tower |
| 200 ft | About 20 miles | Repeater on a high tower |

Rule of thumb: radio horizon in miles ≈ 1.41 × √(height in feet). Add both stations' horizons for the most you could hope for over flat ground.

## Putting it together

- ≈7 — miles: two handhelds on flat ground, at best

- ≈11 — miles: handheld to a 30 ft ranch mast

- ≈23 — miles: handheld to a 200 ft repeater

- 1–2 — miles: real life in hills and trees

These are best-case figures over flat, open land. Arkansas hills and forest cut them down sharply.

## Remember

Before buying more power, get the antenna higher.