# Antennas 101

The cheapest way to double what your radio can do

## Four kinds of GMRS antennas

| Type | Where it goes | What it does |
| --- | --- | --- |
| Stock "rubber duck" | On the handheld | Short and tough, but the weakest |
| Long whip (15–18 in) | On the handheld | Noticeably better range, still portable |
| Magnet-mount | Truck roof | Uses the metal roof as a ground plane; big gain over a handheld inside a cab |
| Base antenna (J-pole or similar) | Mast or roofline | Highest and best; the backbone at the ranch |

## Getting it right

- Match the connector: UV-5G Plus takes SMA-female antennas; check before ordering

- Mount it high: Every foot of height helps more than extra watts

- Keep cable short: Long, thin coax wastes signal; use quality low-loss cable

- Never transmit without it: Talking with no antenna attached can damage the radio

Source: True Prepper: Best GMRS radios

## Inside a vehicle

### Handheld in the cab

- Metal body blocks much of the signal

- Range can drop by half or more

- Hold it near a window at least

### Roof antenna outside

- Signal goes out in every direction

- Pair with a mobile radio up to 50 W

- Best upgrade for the drive to the ranch

## Remember

A $25 antenna placed high often beats a $200 radio placed low.