# Making Batteries Last a Week

In a long outage, battery life matters more than range

## Where the battery goes

### Transmitting

- Uses the most power by far

- High power drains it fastest

- Short messages save a lot

### Listening

- Uses far less

- But it adds up when left on all day

- The screen light uses some too

## Stretch every charge

- Listen on a schedule: First 10 minutes of each hour instead of all day

- Use Low power up close: Save High for long distance

- Talk less: Plan what to say before pressing the button

- Dim the screen: Shorten the backlight time in the menu

## Ways to recharge

| Source | How | Notes |
| --- | --- | --- |
| Wall outlet | USB-C cable | Top off whenever there's power |
| Power station | USB port | One 1,000 Wh unit can charge a radio dozens of times |
| Vehicle | USB car adapter | Charge while driving to the ranch |
| Solar | Small USB panel | Slow but endless |
| Spare battery | Swap in | Keep one charged in the go bag |

## Remember

Two charged batteries and a schedule beat one battery left on all day.