# Repeaters: Offsets and Tones

How a mountaintop radio turns your 5 watts into 50 miles

# How a repeater works

- You transmit on its input: Usually 600 kHz away from the output on 2 meters

- It checks your tone: The right CTCSS tone opens the repeater

- It retransmits on its output: From a tall tower with more power

- Everyone in range hears you: Across a whole county or more

Offsets: 2 meters: 600 kHz, plus or minus. 70 centimeters: 5 MHz, usually plus.

# Repeaters to program first

| Repeater | Output | Offset | Tone |
| --- | --- | --- | --- |
| W4CAT Nashville | 146.955 | + | 114.8 |
| K5PXP Mount Nebo (ranch) | 146.820 | – | 131.8 |
| N4OEM Nashville ARES | 442.750 | + | 100.0 |
| N5XMZ Magazine Mountain | 443.250 | + | 114.8 |

Listings change. Check RepeaterBook before you rely on one, and listen first.

# Good repeater manners

- Listen first: Make sure no one is using it

- Pause between turns: Leave room for someone with an emergency

- Say your call sign: 'W4JUM monitoring' is all it takes

- Keep it short: Many repeaters time out after a few minutes

Sources: RepeaterBook listings; ARRL repeater guidance

# Remember

Right frequency, right offset, right tone; then listen before you talk.

# Test yourself

- What is the standard repeater offset on 2 meters?

a) 600 kHz

b) 5 MHz

c) 1 MHz

d) No offset

- What is the usual offset on 70 centimeters?

a) 600 kHz

b) 5 MHz

c) 50 kHz

d) 20 MHz

- Why do most repeaters need a tone?

a) To play music

b) To open the repeater only for intended users

c) To charge your radio

d) To encrypt your voice

- What should you do before keying up on a repeater?

a) Talk right away

b) Listen to make sure it's not in use

c) Turn power to low

d) Change your call sign

- Why pause between turns on a repeater?

a) To save battery

b) So someone with an emergency can break in

c) Because it's the law to wait 1 minute

d) To reset the tone

# Answers

1. 600 kHz. 2-meter repeaters use a 600 kHz split.

2. 5 MHz. 70 cm repeaters usually use a 5 MHz split.

3. To open the repeater only for intended users. The CTCSS tone keeps stray signals from triggering the repeater.

4. Listen to make sure it's not in use. Always listen first.

5. So someone with an emergency can break in. A short pause lets others, especially emergencies, get in.