# Simplex: Radio to Radio

When the repeater is down, talk straight to each other

# Key simplex frequencies

- 146.520 — 2 m national calling

- 446.000 — 70 cm national calling

- 1–3 — miles, handheld to handheld

- 0 — towers needed

Call on the calling frequency, then move to another simplex channel to keep talking.

# A family simplex plan

- Pick a main channel: For example 146.550

- Pick a backup: For example 146.580, in case the main is busy

- Save both in memory: Name them FAM1 and FAM2

- Practice monthly: Test from home, the truck and the ranch

Why it matters: Repeaters can lose power in a storm. Simplex works as long as your batteries do.

# How far simplex reaches

| Setup | Typical range |
| --- | --- |
| Handheld to handheld, in town | 1 to 3 miles |
| Handheld from a hilltop | 5 to 15 miles |
| Truck radio to truck radio | 10 to 20 miles |
| Base antenna to base antenna | 20 to 40 miles |

Estimates for open terrain; hills and trees cut range sharply.

Sources: ARRL band plan; typical VHF line-of-sight estimates

# Remember

Every family plan needs a simplex channel that works with no tower at all.

# Test yourself

- What does simplex mean?

a) Talking through a repeater

b) Talking radio to radio on one frequency

c) Sending text messages

d) Using two radios at once

- What is the 2-meter national simplex calling frequency?

a) 146.520

b) 146.955

c) 462.675

d) 27.185

- After making contact on a calling frequency you should:

a) Stay there all day

b) Move to another simplex frequency

c) Turn off your radio

d) Switch to a repeater

- Why does a family need a simplex plan?

a) Repeaters can fail in a storm

b) Simplex is louder

c) Simplex needs no batteries

d) It's required by law

- About how far do two handhelds reach in town?

a) 100 miles

b) 1 to 3 miles

c) 50 feet

d) Unlimited

# Answers

1. Talking radio to radio on one frequency. Simplex is direct, same-frequency communication.

2. 146.520. 146.520 MHz is the 2 m calling frequency.

3. Move to another simplex frequency. Keep the calling frequency clear for others.

4. Repeaters can fail in a storm. Simplex works without towers or grid power.

5. 1 to 3 miles. Handheld to handheld usually covers a few miles.