# Antennas: The Best Upgrade

A better antenna beats more power every time

# Antenna choices

| Antenna | Where | Improvement |
| --- | --- | --- |
| Stock rubber duck | On the radio | Baseline |
| Long whip | On the radio | Noticeably better |
| Magnet mount | Truck roof | Large improvement |
| Roof or mast antenna | House or ranch shed | The biggest gain |

Height and a clear view beat everything else.

# Things to know

- Get it high: Every few feet of height adds miles

- Short, good coax: Cheap cable loses much of your signal at UHF

- Check SWR: A meter tells you if the antenna is matched

- Never transmit without one: It can damage the radio

# Simple home-built options

J-pole

- Easy to build from copper pipe or ladder line

- Good on 2 meters

- Mounts on a mast or tree

Ground plane

- A few wires and a connector

- Cheap and effective

- A classic first project

Sources: ARRL Antenna Book; general VHF/UHF practice

# Remember

Antenna height and quality matter more than watts.

# Test yourself

- What usually improves range the most?

a) More watts

b) A higher, better antenna

c) A louder voice

d) A new battery

- What does an SWR meter check?

a) Battery charge

b) How well the antenna is matched

c) Your call sign

d) The weather

- Why choose good coax at UHF?

a) It's prettier

b) Cheap coax loses much of the signal

c) It's required by law

d) It charges the radio

- What should you never do?

a) Transmit without an antenna

b) Use a magnet mount

c) Raise the antenna

d) Use a J-pole

- Which is a classic easy home-built 2-meter antenna?

a) J-pole

b) Satellite dish

c) TV rabbit ears

d) Car horn

# Answers

1. A higher, better antenna. Antenna height and quality matter most.

2. How well the antenna is matched. Low SWR means power goes out the antenna instead of back into the radio.

3. Cheap coax loses much of the signal. Loss in thin coax rises at higher frequencies.

4. Transmit without an antenna. Transmitting without an antenna can damage the radio.

5. J-pole. J-poles are simple and effective.