# Power When the Grid Is Down

A radio is only as good as its battery

# Power options

| Source | Runs | Notes |
| --- | --- | --- |
| Radio battery | About a day | Carry a charged spare |
| AA battery case | Hours to a day | Use lithium AAs in cold |
| Truck 12 V outlet | As long as you have fuel | Don't drain the starting battery |
| Solar and a battery bank | Days to weeks | The ranch system can run a base station |

Receiving uses little power; transmitting on high uses the most.

# Stretch your battery

- Use low power: Switch to high only when needed

- Listen on a schedule: Check in at set times instead of all day

- Keep it warm: Cold batteries lose capacity

- Rotate spares: Charge and test them every month

# At the ranch

What you have

- Solar panels and a LiFePO4 battery bank

- A 12-volt or 24-volt system

- Space for a base antenna

What to add

- A 12 V DC supply or converter for the radio

- A fused power lead

- A mast antenna on the shed

Sources: ARRL emergency power guidance; general battery practice

# Remember

Carry spares, transmit on low, listen on a schedule, and let the sun recharge the rest.

# Test yourself

- Which uses the most battery?

a) Listening

b) Transmitting on high power

c) Turning off the radio

d) Reading the screen

- Why use lithium AAs in cold weather?

a) They're cheaper

b) They hold up better in the cold

c) They're required

d) They charge themselves

- How can you stretch battery life in an outage?

a) Talk constantly

b) Check in on a schedule

c) Use the flashlight

d) Turn up the volume

- What can power a base station at the ranch for days?

a) A solar system with a battery bank

b) A single AA

c) Nothing

d) A phone charger

- What should every radio lead from a battery have?

a) A fuse

b) A light

c) A switch shaped like a star

d) Nothing

# Answers

1. Transmitting on high power. Transmitting on high draws the most current.

2. They hold up better in the cold. Cold weakens most batteries; lithium handles it better.

3. Check in on a schedule. Scheduled check-ins save power.

4. A solar system with a battery bank. Solar and batteries can run a radio through long outages.

5. A fuse. Fuses protect against shorts and fires.