# Upgrading to General

Talk across the country without any tower at all

# The General test

- 35 — questions

- 26 — to pass

- 0 — extra FCC fee for an upgrade

- HF — bands opened

Many exam sessions let you try the next test the same day at no extra cost.

# How HF reaches so far

- Your signal goes up: Toward the sky at an angle

- The ionosphere bends it back: A charged layer high above the earth

- It lands hundreds of miles away: Sometimes thousands

- It can bounce again: Skipping across the country

Changes with the sun: Day, night and the solar cycle change which bands work.

# A simple HF station

| Piece | Notes |
| --- | --- |
| HF radio | 100 watts is common |
| Wire antenna | A dipole between two trees works well |
| Power supply | About 20 to 30 amps at 12 V |
| Tuner | Helps match the antenna on several bands |

A ranch with tall trees is a great place for an HF wire antenna.

Sources: FCC Part 97; ARRL General class materials

# Remember

General opens HF, where the sky itself is your repeater.

# Test yourself

- What does the General license mainly open?

a) HF bands

b) GMRS channels

c) CB channels

d) FM broadcast

- How many questions are on the General test?

a) 10

b) 35

c) 50

d) 100

- What bends HF signals back to earth?

a) The ionosphere

b) The ocean

c) Mountains

d) Clouds

- Which simple antenna works well for HF?

a) A wire dipole between two trees

b) A rubber duck

c) A TV antenna

d) A magnet mount

- What changes which HF bands work?

a) Time of day and the sun

b) Your call sign

c) Your radio's color

d) The day of the week

# Answers

1. HF bands. General adds most HF privileges.

2. 35. 35 questions, 26 to pass.

3. The ionosphere. The ionosphere refracts HF signals.

4. A wire dipole between two trees. Wire dipoles are simple and effective.

5. Time of day and the sun. Propagation depends on the sun and time of day.